# METHOD OF STATEMENT

Date: 02nd Feb 2018

Description of Work

To unload 3 cases of 1 unit TP-32 Wire Cut Machine from lorry chassis, shift and position to 5th floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

08/02/2018   -   09:45 am

Location of Operation

#### 4 Fusionopolis Way Kinesis Tower #05-03

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Lorry crane to park at designated hoisting area

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Insert pallet truck to machine base for the move

- Transport machine to 5th floor via cargo lift

- Shift and position machine to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175