# METHOD OF STATEMENT

Date: 02nd June 2022

Description of Work

To unload and hoist down 5 cases of 5 unit vacuum pump and accessories from lorry chassis to the ground

Machine Spec

VADS 1500 Vacuum Pump

Dim: 170 x 150 x 180 cm / Wt: 1, 400 kg each

3-4 pallet accessories

Dim: 120 x 80 x 80 cm / Wt: 250 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

15-30/06/22

Time of Operation

10:00 am

Location of Operation

#### 133 Tuas South Ave 3

Ciba Vision Asian Manufacturing & Logistics Pte Ltd

Equipment Use

633 lorry loader

1 set wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert sling to crane hook and secured onto case rigging point

- Lorry loader to hoist up case from lorry chassis

- Swing crane and lower case to near ground level

- Unload case at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175