# METHOD OF STATEMENT

Date: 14th June 2023

Description of Work

To unload 1 unit machine from lorry chassis, shift and transport to 5th floor via cargo lift

Machine Spec

1 unit Brother S700X2

Dim: 205 x 200 x 205 cm

Wt: 2, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17/06/23 (Sat)

Time of Operation

09:40 am

Location of Operation

#### 16 Ayer Rajah Crescent

Tempco Manufacturing (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert sling to crane hook and secured onto machine rigging point

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine to 5th floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175