# METHOD OF STATEMENT

Date: 16th Dec 2021

Description of Work

To unload 3 pallet of 2 unit machine from lorry chassis, shift and transport into premise at ground floor

Machine Spec

2 unit Yamasaki Giken / YMZ-850

Dim: 330 x 228 x 254 cm

Wt: 4, 950 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

18/12/21 (Sat)

Time of Operation

09:45 am

Location of Operation

#### 3 Toh Guan Rd East

Knight Auto Precision Pte Ltd

Equipment Use

2 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated unloading area

- Insert sling to crane hook and secure onto machine hoisting point

- Hoist down machine from lorry chassis to near ground level

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175