# METHOD OF STATEMENT

Date: 20th Mar 2020

Description of Work

To unload 1 case from lorry chassis to the ground, uncrate, hoist up and detach machine from base, shift and position to 2nd floor via cargo lift

Machine Spec

Explorer Performance 08.10.06 CMM

Dim: 180 x 130 cm / Wt: 1, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/03/2020

Time of Operation

Afternoon schedule

Location of Operation

#### 1 Joo Koon Crescent

Sunningdale Tech Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Hoist down base to the ground

- Insert sling to machine hoisting point

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and swing crane away from base

- Lower down machine to near ground level

- Insert pallet truck onto machine base

- Shift and transport machine to 2nd floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175