# METHOD OF STATEMENT

Date: 30th Jan 2020

Description of Work

To hoist up 1 unit CMM machine to 2nd level shutter gate opening by using lorry loader

Equipment Spec

Global S 05.07.05

Case Dim: 3038 x 1288 x 2239

G/Wt: 1, 118 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03/02/2020

Time of Operation

09:45 am

Location of Operation

#### 1 Joo Koon Crescent

Sunningdale Tech Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Uncrate machine case

- Insert sling to machine hoisting point

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up machine to 2nd floor door opening

- Lower down machine at 2nd floor

- Insert pallet truck onto machine stand

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175