# METHOD OF STATEMENT

Date: 19th Feb 2020

Description of Work

To hoist up 1 unit Mold to 2nd level shutter gate opening by using lorry loader

Mold Spec

Dim: 70 x 70 x 70 cm

Wt: 430 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/02/2020

Time of Operation

11:00 am

Location of Operation

#### 1 Senoko South Rd

Sumitomo Bakelite (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert sling to mold hoisting point

- Lorry loader to hoist up mold 1ft above ground to check on balancing

- When well balanced, hoist up mold to 2nd floor door opening

- Lower down mold at 2nd floor

- Insert pallet truck onto mold base

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175