# METHOD OF STATEMENT

Date: 02nd June 2020

Description of Work

To unload 1 unit machine from lorry chassis to the ground, shift and position into production

Machine Spec

1 unit T-Jaw Band Saw

Dim: 115 x 70 x 212 cm

Wt: 426 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

09/06/2020

Time of Operation

10:00 am

Location of Operation

#### 1800 West Camp Rd, Seletar Aerospace Pk

Fokker Services Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert webbing sling to machine hoisting point

- Hoist down machine from lorry chassis to the ground

- Insert pallet truck under machine base

- Shift and transport machine into production

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175