# METHOD OF STATEMENT

Date: 08th Jan 2020

Description of Work

To hoist up 1 unit Equipment to 2nd level shutter gate opening by using lorry loader

Equipment Spec

Dim: 1200 x 550 x 1320 mm

Wt: 200 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

09/01/2020

Time of Operation

Afternoon schedule

Location of Operation

#### NUS, 2 Engineering Dr 2, EW1-04-01

Department of Civil & Transportation Engineering

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert sling onto equipment hoisting point

- Lorry loader to hoist up equipment 1ft above ground to check on balancing

- When well balanced, hoist up to 2nd level

- Lower down equipment near shutter door opening

- Insert pallet truck onto equipment base

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

REMARK:

Customer to dismantle railing at 2nd level

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175