# METHOD OF STATEMENT

Date: 08th Nov 2022

Description of Work

To hoist up 1 unit machine to 2nd level by using lorry loader

Machine Spec

Dim: 220 x 100 x 85 cm

Wt: 440 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

15/11/22

Time of Operation

Afternoon schedule

Location of Operation

#### NUS, 2 Engineering Dr 2, EW1-04-02

Department of Civil & Environmental Engineering Transportation Lab

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert sling to crane hook and secure to rigging point of machine

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up to 2nd level of building

- Lower down machine to near ground level

- Insert pallet truck under machine base

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175