# METHOD OF STATEMENT

Date: 31st Mar 2020

Description of Work

To hoist up 1 unit machine to 2nd floor through shutter gate opening by using lorry loader

Machine Spec

1 unit CSP-45 Single Crank Power Press

Dim: 1075 x 1320 x 2320 mm

Wt: 3, 450 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/04/2020

Time of Operation

10:00 am

Location of Operation

#### 51 Joo Koon Circle

Omni Mould Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire sling

Shackle

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc…safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Attached lifting gears to crane hook and secured to hoisting point of machine to be lifted

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up machine to 2nd floor shutter door opening

- Lower down machine to near ground level

- Insert rollers under machine base

- Shift and position machine to the designated location

- Unload machine from rollers and insert levelling pad under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175