# METHOD OF STATEMENT

Date: 01st Dec 2021

Description of Work

1) To unload 1 unit machine from lorry chassis, shift and position into factory premise

2) To shift out 1 unit old grinder from production to loading bay, hoist and load up onto customer arrange lorry

Machine Spec

Move in

1 unit Mitsui 250MHD Grinder

Dim: 220 x 181 x 203 cm / Wt: 1, 250 kg

Move out

1 unit old Grinder

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/01/22

Time of Operation

09:45 AM

Location of Operation

#### 60 Tuas Ave 11

Panduit (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert sling to crane hook and secured onto hoisting point of machine

- Hoist down machine from lorry chassis to near ground level

- Insert pallet truck under machine base

- Shift and transport machine into factory premise

- Position machine at the designated location

- To move out 1 unit old grinder from production

- Shift out to loading bay

- Hoist up and load onto customer arrange lorry

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175