# METHOD OF STATEMENT

Date: 04th May 2020

Description of Work

To unload 1 unit machine from lorry chassis, shift and position into premise at ground floor

Machine Spec

1 unit Mitsui MSG250MH

Dim: 1850 x 1310 x 1800 mm

Wt: 1, 100 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est: 20/05/2020

Time of Operation

10:00 am

Location of Operation

#### 60 Tuas Ave 11

Panduit

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert webbing sling to machine hoisting point

- Hoist down machine from lorry chassis to the ground

- Insert pallet truck onto machine base

- Shift and transport machine into premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175