# METHOD OF STATEMENT

Date: 16th June 2020

Description of Work

To hoist down and unload 1 unit machine from lorry chassis to the ground, shift and position into production

Machine Spec

1 unit Band Saw / H-260HB

Dim: 2170 x 2100 mm

Wt: 1, 750 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

24/06/2020

Time of Operation

11:00 am

Location of Operation

#### 7 Joo Koon Circle

Honeywell (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Insert sling to machine hoisting point

- Hoist down machine from lorry chassis to the ground

- Insert rollers/pallet truck under machine base

- Shift and transport machine into production

- Position machine at the designated location

- Unload form rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175