# METHOD OF STATEMENT

Date: 20th May 2024

Description of Work

To hoist down 1 unit Bar Feeder from 2nd floor by using lorry loader

Bar Feeder Spec

433 x 41 x 112 cm

Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est:

23-24/05/24

Time of Operation

Afternoon schedule

Location of Operation

#### Blk 20 Woodlands Link #02-

Fyno Precision Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck/rollers

Webbing sling/shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Setting up of lorry loader at site location

- Shift out bar feeder to near crane parking area

- Insert webbing sling to crane hook and secure to rigging point of bar feeder

- Lorry loader to hoist up bar feeder 1ft above ground to check on balancing

- When well balanced, hoist up bar feeder above parapet wall

- Crane to slowly swing bar feeder out

- Lower down bar feeder and load onto lorry chassis

- Secure with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175