# METHOD OF STATEMENT

Date: 15th Nov 2022

Description of Work

1) To hoist down 3 unit Bar Feeder from 2nd floor by using lorry loader

2) To shift out 4 unit machine from 2nd floor to loading bay via cargo lift and load onto Malaysia   lorry

Bar Feeder Spec

400 x 60 x 110 cm

Wt: 300 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

26/11/22 (Saturday)

Time of Operation

09:45 am

Location of Operation

#### Blk 2024 Bukit Batok St 23 #02-84

Rigid Precision Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck/rollers

Webbing sling/shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deployment of lorry loader at designated hoisting area

- Setting up of lorry loader at site location

- Shift out bar feeder to near crane parking area

- Insert required lifting gears to crane hook and secure to rigging point of bar feeder

- Lorry loader to hoist up bar feeder 1ft above ground to check on balancing

- When well balanced, hoist up bar feeder above parapet wall

- Crane to slowly swing bar feeder out

- Lower down bar feeder and load onto Malaysia lorry

- Secure with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175