# METHOD OF STATEMENT

Date: 26th May 2023

Description of Work

To hoist down 1 unit equipment from 2nd floor by using platform and lorry loader

Machine Spec

1 unit Assembly machine

Dim: 200 x 156 x 150 cm

Wt: 900 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

05/03/24

Time of Operation

10:00 am

Location of Operation

#### Blk 2023 Bukit Batok St 23 #03-92/96

Pocket Technology Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x platform

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deployment of lorry loader at designated hoisting area

- Setting up of lorry loader at site location

- Shift out equipment to near crane parking area

- Insert required lifting gears to crane hook and secure to rigging point of equipment

- Lorry loader to hoist up equipment 1ft above ground to check on balancing

- When well balanced, hoist up equipment above parapet wall

- Crane to slowly swing equipment out

- Lower down equipment and load onto lorry chassis

- Secure with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175