# METHOD OF STATEMENT

Date: 24th Aug 2023

Description of Work

To hoist down 1 unit machine from 4th floor and hoist up 1 unit machine to 4th floor by using lorry loader

Machine Spec

Lathe machine

Dim: 224 x 98 x 121 cm

Wt: 1, 700 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/09/23

Time of Operation

09:30 am

Location of Operation

#### Blk 3015 Ubi Rd 1 #04-256

Meng Hoi Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck/rollers

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Setting up of lorry loader at site location

- Shift out bar machine to near crane parking area

- Insert webbing sling to crane hook and secure to rigging point of machine

- Lorry loader to hoist up machine1ft above ground to check on balancing

- When well balanced, hoist up machine above parapet wall

- Crane to slowly swing machine out

- Lower down machine and load onto forwarder lorry

- Secure with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175