# METHOD OF STATEMENT

Date: 26th Mar 2024

Description of Work

To hoist down 4 unit machine from 2nd floor by using lorry loader

Machine Spec

2 unit 5HP Milling

2 unit 7m Lathe

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

01/04/24

Time of Operation

10:00 am

Location of Operation

#### Blk 2021 Bukit Batok St 23 #02-192

Weldbits Engineering Works

Equipment Use

1-2 x 633 lorry loader

Pallet truck/rollers

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deployment of lorry loader at designated hoisting area

- Setting up of lorry loader at site location

- Shift out machine to near crane parking area

- Insert required lifting gears to crane hook

- Secure to rigging point of machine

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up machine above parapet wall

- Crane to slowly swing machine out

- Lower down machine to the ground

- Load up onto lorry chassis

- Secure machine with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175