# METHOD OF STATEMENT

Date: 22nd Apr 2024

Description of Work

To hoist down 1 unit machine from 2nd floor by using lorry loader

Machine Spec

1 unit Lathe machine

Dim: 220 x 100 x 130 cm / Wt: 1, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

04/05/24

Time of Operation

06:00 am

Location of Operation

#### Blk 3007 Ubi Rd 1 #02-456

Alson Engineering (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck/rollers

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deployment of lorry loader at designated hoisting area

- Setting up of lorry loader at site location

- Shift out machine to near crane parking area

- Insert required lifting gears to crane hook

- Secure to rigging point of machine

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up machine above parapet wall

- Crane to slowly swing machine out

- Lower down machine to the ground

- Load up onto lorry chassis

- Secure machine with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

REMARK:

This job scope will only proceed if machine unable to turn into unit #02-458 and be hoisted through wall hacking opening

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175