# METHOD OF STATEMENT

Date: 22nd Sep 2022

Description of Work

To hoist up 1 unit new machine and hoist down 1 unit old machine at 4th floor by using lorry loader

Machine Spec

Hoist up

1 unit Equiptop 1224 Surface Grinder

Dim: 266 x 193 x 180 cm

Wt: 1, 900 kg

Hoist down

1 unit Troop Surface Grinder

Dim: 260 x 193 x 180 cm

1, 850 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/10/22

Time of Operation

09:45 am

Location of Operation

#### Blk 2022 Bukit Batok St 23 #04-118

Jack & Lee Engineering

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deployment of lorry loader at designated hoisting area

- Setting up of lorry loader at site location

- Insert required lifting gears to crane hook and secure to machine rigging point

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up machine 4th floor

- Swing crane and lower down machine onto corridor pathway

- Insert pallet truck onto machine base

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175