# METHOD OF STATEMENT

Date: 26th Oct 2020

Description of Work

To hoist up machineries to 2nd floor by using lorry loader

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

26/10/2020 – 01/11/2020

Time of Operation

10:00 pm

Location of Operation

#### Blk 2024 Bukit Batok St 23 #02-28

Micronvision Precision (S) Pte Ltd

Equipment Use

2-3 x 633 lorry loader

Pallet truck

1 set wire sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deployment of lorry loader at designated hoisting area

- Setting up of lorry loader at site location

- Insert required lifting gears to crane hook and secure to machine hoisting point

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up slowly to 2nd floor

- Swing crane and lower down machine onto corridor pathway

- Insert pallet truck onto machine base

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175