# METHOD OF STATEMENT

Date:

Description of Work

To hoist up by using lorry loader

Machine Spec

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

09:45 am

Location of Operation

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deployment of lorry loader at designated hoisting area

- Setting up of lorry loader at site location with outrigger fully extended and resting on steel plate/timber mats as base plate

- Rigger to insert sling to crane hook

- Secure sling to rigging point of bar feeder

- Lorry loader to hoist up bar feeder 1ft above ground to check on balancing

- When well balanced, hoist up bar feeder 2nd floor

- Swing crane and lower down bar feeder into corridor pathway

- Insert pallet truck onto bar feeder stand

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175