# METHOD OF STATEMENT

Date: 11th Apr 2022

Description of Work

To unload 2 unit machine from Malaysia lorry, shift and transport into factory premise at ground floor

Machine Spec

1 unit Surface Grinder

Dim: 115 x 115 x 185 cm

Wt: 820 kg

1 unit Supermax Milling

Dim: 175 x 190 x 225

Wt: 1, 289 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

11/04/22

Time of Operation

Afternoon schedule

Location of Operation

#### 4 Clementi Loop #01-01

Santak Metal Manufacturing Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated hoisting area

- Reverse Malaysia lorry to near crane

- Insert sling to crane hook and secured onto machine hoisting point

- Lorry loader to hoist up machine from Malaysia lorry

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175