# METHOD OF STATEMENT

Date: 09th Oct 2017

Description of work

Lorry crane to hoist up and load machine onto lorry/low loader chassis and transfer to new factory

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

09/10/2017   -   09:30 am

Location of Operation

#### 23 Woodlands Terrace

Metform Industries Pte Ltd

Equipment Use

1 x 75 ton lorry crane

1 x 50 ton lorry crane

Lorry crane

Low loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Drive lorry crane and low loader into customer premise

- Hoist up and load machine onto low loader/lorry chassis

- Secured machine with strapping belt prior to moving out of premise and transport to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175