# METHOD OF STATEMENT

Date: 22nd Jan 2025

Description of work

1) To unload 1 unit machine from lorry chassis using loading bay, shift and position into tenant premise at 5th floor via cargo lift

2) To shift out 1 unit machine from 5th floor to loading bay via cargo lift and load up onto lorry chassis

Machine Spec

Move in

1 unit Kinwa CNC Lathe

Wt: 3, 300 kg

Move out

1 unit Goodway GLS-200 CNC Lathe

Wt: 3, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/02/25

Time of Operation

13:00 pm

Location of Operation

#### Blk 1 A.M.K Ind Pk 2A #05-12

Equipment Use

1 x 633 lorry loader

Rollers

Pallet truck

Woodblock

Crowbar/hand jack

Obstacles

No obstacle at loading/unloading bay

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Environment

# No operation will be carry out on heavy rain, thunderstorms and lighting weather

# All debris will be clear and disposed

# JOB SCOPE

- Deploy lorry crane into site

- Reverse lorry to loading bay

- Jack up machine using hand jack

- Insert pallet truck under machine bed

- Shift out machine from lorry chassis

- Transport machine into tenant premise at 5th floor via cargo lift

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175