# METHOD OF STATEMENT

Date: 24th Feb 2025

Description of work

To shift out 3 units machine from production to loading bay, hoist up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

1 unit Deflashing machine

Dim: 110 x 100 x 210 cm / Wt: 800 kg

1 unit Degreasing machine

Dim: 330 x 130 x 170 cm / Wt: 1, 500 kg

1 unit used machine

Dim: 85 x 55 x 235 cm / Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/02/25

Time of Operation

09:30 am

Location of Operation

#### 10 Tuas South St 5

CDI Energy Products Pte Ltd

Equipment Use

1 x 633 lorry loader

Crowbar

Woodblock

Rollers/pallet truck

1 set wire sling

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert pallet truck under machine base

- Shift out machine from production to loading bay

- Riggers to insert sling to crane hook

- Secured sling to rigging point of load to be lifted

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto lorry chassis

- Release all sling from rigging point of machine

- Secured machine with strapping belt prior to moving out of customer premise

- Transport machine to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175