# METHOD OF STATEMENT

Date: 19th Dec 2025

Description of work

To shift out 1 unit press machine from production to loading bay, hoist and load up onto low bed

Spec

Hydraulic Press machine

Dim: 250 x 130 x 330 cm

Wt: 3, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBC

Time of Operation

TBC

Location of Operation

21 Loyang Crescent

Safran Landing Systems Services Singapore P/L

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Hand jack/crowbar

Woodblock

Wire sling

Obstacles

No obstacle on loading/unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

# No operation will be carry out on heavy rain, thunderstorms and lighting weather

# All debris will be clear and disposed

# Safety Aspect

# All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place

# All our workers are fully equipped with standard P.P.E (safety shoe, safety helmet)

# Lifting machine and lifting gear with M.O.M certificate

# M.O.M certified lifting supervisor, rigger, signalman and lorry loader will be involved in the operation

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- To shift out machine from ground floor to loading area

- Rigger to insert sling to lorry crane hook

- Secure sling to rigging point of machine

- Hoist up machine and load onto customer arrange low-bed

- Low-bed driver to secured machine with belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175