# METHOD OF STATEMENT

Date: 14th Jan 2022

Description of work

To shift out 1 unit machine and accessories from production to loading bay, hoist up onto low loader chassis and deliver to another location

Machine Spec

1 unit Wele UG800 CNC

Dim: 500 x 330 x 280 cm

Wt: 12, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/01/22

Time of Operation

09:45 am

Location of Operation

#### Blk 16B Tuas Ave 1 #02-40/42

Rohag

Equipment Use

2 x 633 lorry loader

1 x low loader

Hand jack/woodblock

Rollers/pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine base

- Shift out machine from production to loading bay

- Riggers to secured sling to rigging point of load to be lifted

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto low loader

- Release all sling from rigging point of machine

- Secured machine with strapping belt prior to moving out of customer premise

- Transport machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175