# METHOD OF STATEMENT

Date: 21st Aug 2021

Description of work

To hoist up machine and accessories onto Malaysia lorry

Machine Spec

Kyori Press Beat 40N

Dim: 2100 x 1200 x 2800 mm

Wt: 6, 000 kg

Sound Enclosure

Dim: 3300 x 1950 x 1000 mm

Wt: 1000 kg

Cooling unit control panel

Dim: 1600 x 1100 x 1600 mm

Wt: 700 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

23/09/21

Time of Operation

09:45 am

Location of Operation

#### 6 Gul St 5

FCI Connectors

Equipment Use

1 x 633 lorry loader

1 x Malaysia lorry

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting bay

- Riggers to secured sling to rigging point of load to be lifted

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto Malaysia lorry

- Release all sling from rigging point of machine

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175