# METHOD OF STATEMENT

Date: 13th Apr 2020

Description of work

To shift out 4 unit machine from production to loading area, hoist up onto lorry chassis and deliver to another location

Machine Spec

1 unit 60ton Press

1 unit 80ton Press

1 unit Grinder

1 unit Shearing

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17/04/2020

Time of Operation

09:45 am

Location of Operation

No 30,32 Tuas Ave 8

ASM

Equipment Use

1-2 x 633 lorry loader

Hand jack/crowbar woodblock

Rollers/pallet truck

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert rollers/pallet truck under machine base

- Shift out machine from production to loading bay

- Attached sling to crane hook and secured onto machine to be lifted

- Hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise and deliver to another location

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175