# METHOD OF STATEMENT

Date: 08th Sep 2020

Description of work

To shift out 2 unit machine and accessories from production to loading bay, hoist up onto lorry chassis and deliver to Eric Wong warehouse

Machine Spec

1 unit Makino EDNC 65

Dim: 320 x 250 x 280 cm / Wt: 6, 400 kg

1 unit Makino Max 65

Dim: 240 x 220 x 230 cm / Wt: 4, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date Of Operation

09/09/2020 (Wed)

Time Of Operation

09:45 am

Location of Operation

#### 5 Tuas Dr 1

Fu Yu Corporation Limited

Equipment Use

2 x 633 lorry loader

Hand jack/woodblock

Rollers/pallet truck

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers/pallet truck under machine base

- Shift out machine from production to loading bay

- Riggers to secured sling to rigging point of load to be lifted

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Transport machine to Eric Wong warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175