# METHOD OF STATEMENT

Date: 11th June 2020

Description of work

To shift out 2 unit machine with accessories from production room to loading area, hoist and load onto wooden base for packing, after packing load onto low loader

Machine Spec

Global 20.33.15 CMM Machine

Case dim: 460 x 280 x 90 cm

Wt: 14, 600 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

01-31/07/2020

Time of Operation

09:30 am

Location of Operation

FMC

11 Jalan Nip 1/1A Taman Industri Nusa Jaya

Equipment Use

1 x 50ton mobile crane

1 x lorry crane

1 x forklift

1 set rollers

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine bed

- Shift out machine from production area

- Transport machine to designated hoisting/packing area

- Lay out wooden base for packing

- Attached lifting gears to crane hook and secured to hoisting point of machine to be lifted

- Mobile crane to hoist up machine and load onto wooden base for packing

- Pack to form a case

- After packing, hoist up case and load onto low bed

- Secured case with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175