# METHOD OF STATEMENT

Date: 09th May 2025

Description of work

To shift out 1 unit machine with accessories from production room to loading area, using forklift to fork up machine and load onto wooden base for packing, after packing stuff onto container

Machine Spec

1 unit Mazak M5NRB Machine

Dim: 5.77 x 2.14 x 2.14 m

Wt: 7, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

13/05/25

Time of Operation

09:45 am

Location of Operation

#### No 11 Tuas South Ave 12

#### Halliburton Completions MFG Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 10 ton forklift

1 x low bed

Hydraulic jack

Woodblock

1 set air rollers

Rollers/pallet truck

Wire/webbing sling

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator and forklift driver will be involved in this operation

Safety Aspect

Access to be restricted to the site area, be cordoned off with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

# JOB SCOPE

- Jack up machine to 3” high

- Insert air rollers under machine bed

- Shift out machine from production area

- Transport machine to designated hoisting/packing area

- Lay out wooden base for packing

- 10ton forklift to fork up machine

- Load machine onto wooden base for packing

- Pack to form a case

- After packing, fork up case and load onto customer arrange container

- Secured case with choking wood and lashing wire

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175