# METHOD OF STATEMENT

Date: 07th November 2024

Description of work

To shift out 4 unit machine with accessories from production room to loading area, hoist and load onto wooden base for packing, after packing stuff onto FR container

Machine Spec

1 unit Mazak M5NRB Machine

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

05/02/24  –  to shift out accessories for packing

06-07/02/24  –  to shift out main machine for packing

Time of Operation

09:45 am

Location of Operation

#### No 11 Tuas South Ave 12

#### Halliburton Completions MFG Pte Ltd

Equipment Use

1 x 70ton mobile crane

633 lorry loader

1 x 3 ton battery forklift

1 x boom lift

1 set air rollers

Rollers/pallet truck

Hydraulic jack

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator and will be involved in this operation

Safety Aspect

Access to be restricted to the site area, be cordoned off with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

All lifting equipment and accessories selected for the operation is to be assessed prior to use for appropriate and up to date maintenance and inspection records, examination and testing certification

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert air rollers under machine bed

- Shift out machine from production area

- Transport machine to designated hoisting/packing area

- Lay out wooden base for packing

- Riggers will attached sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine and load onto wooden base for packing

- Pack to form a case

- After packing, hoist up case and load onto container

- Secured case with choking wood and lashing wire

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175