# METHOD OF STATEMENT

Date: 14th Sep 2022

Description of work

To hoist up 4 cases onto FR container

Machine Spec

1 unit QTN-400M with accessories

Dim: 525 x 252 x 210 cm

Wt: 16, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

19/09/22

Time of Operation

09:45 am

Location of Operation

#### No 11 Tuas South Ave 12

#### Halliburton Completions MFG Pte Ltd

Equipment Use

1 x lorry loader

1 x 50ton mobile crane

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Access to be restricted to the site area, be cordoned off with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

All lifting equipment and accessories selected for the operation is to be assessed prior to use for appropriate and up to date maintenance and inspection records, examination and testing certification

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Attached sling to crane hook and secure onto rigging point of case

- 1 x 50ton mobile crane to hoist up case 1ft above ground to check on balancing

- When well balanced, hoist up case and load onto container

- Release all sling from case

- Secured case with choking wood and lashing wire

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175