# METHOD OF STATEMENT

Date: 04th June 2020

Description of work

To shift out 2 unit machine with accessories from production room to loading area, hoist and load onto wooden base for packing, after packing load onto FR container

Machine Spec

2 unit Slant Turn Nexus 500

Case dim: 895 x 355 x 385 cm

Wt: 20, 190 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

Halliburton Manufacturing & Technology (M) S/B

22 Jalan Persiaran Teknologi

Equipment Use

1 x mobile crane

1 x lorry crane

1 x forklift

1 set rollers

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Access to be restricted to the site area, be cordoned off with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine bed

- Shift out machine from production area

- Transport machine to designated hoisting/packing area

- Lay out wooden base for packing

- Attached lifting gears to crane hook and secured to hoisting point of machine to be lifted

- Mobile crane to hoist up machine and load onto wooden base for packing

- Pack to form a case

- After packing, hoist up case and load onto container

- Secured machine with choking wood and lashing wire

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175