# METHOD OF STATEMENT

Date: 12th May 2020

Description of work

To shift out/in machine at customer premise

Machine Spec

1 unit 25ton Dobby Press

1 unit 40ton Press

1 unit 80ton Press

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13/05/2020 – 25 ton Dobby Press (to shift out from production and deliver to Eric Wong store)

14/05/2020 – 40 ton Press (to relocate within premise)

15/05/2020 – 80 ton Press (to unload from container, shift and position into production)

Time of Operation

09:45 am

Location of Operation

6 Gul St 5

Amphenol FCI Connectors (S) Pte Ltd

Equipment Use

2 x 633 lorry loader

Hand/hydraulic jack

Woodblock

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

13/05/2020

Jack up machine to 3” high

Insert rollers under machine base

Shift out machine from production to loading bay

Attached sling to crane hook and secured onto machine to be lifted

Hoist up machine and load onto lorry chassis

Secured machine with strapping belt prior to moving out of customer premise

Deliver machine to Eric Wong store

15/05/2020

To unload machine from container

Transport lorry to unloading bay

Hoist down machine from lorry chassis

Insert rollers to machine base

Shift machine into premise

Position machine at the designated location

Unload machine from rollers and insert levelling pad under machine bed

Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175