# METHOD OF STATEMENT

Date: 4th October 2021

Description of work

To shift out 1 unit Datron Neo Machine from NUS 4 Medical Drive via Cargo Lift & Transfer to NUS at Engineering Drive 1 E7. Unload, Shift and position into premises at Level 7 Via Cargo Lift

Spec

1 unit Datron Neo Machine & Vacuum Pump

Wt: 850 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/10/21 (Thursday)

Time of Operation

09:45 am

Location of Operation

#### NUS, 4 Medical Drive #14-01

#### NUS, Engineering Drive 1 E7-07-03

Equipment Use

1 x 633 lorry loader

1 x 2.5 ton forklift

1 set wire/webbing sling

Pallet truck

1 pcs 4ft x 8ft metal plate (to lay out for machine pathway)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down forklift from lorry chassis to the ground

- To shift out machine from workshop to loading area

- Using forklift to fork up machine onto lorry chassis

- Secured machine with strapping belt

- Transport machine to back of building

- To lay out metal plate at stairway for machine pathway

- Fork down machine from lorry chassis and load onto metal plate

- Using pallet truck to shift and position into building premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175