# METHOD OF STATEMENT

Date: 22nd Jan 2025

Description of work

To shift out machine and equipment to loading area, hoist up onto lorry and deliver to new location

Machine spec

1 unit Milling

1 unit Bandsaw

1 unit boltless metal racking

1 unit metal shelf

3 unit metal office cabinet

1 unit work bench

1 unit mini oven

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/02/25

Time of Operation

09:45 am

Location of Operation

#### Addept3D Pte Ltd

#### Move out - 51 Corporation Rd

Move in – 15 Tai Seng Dr

Equipment Use

2 x 633 lorry loader

Crowbar/woodblock

Wire/webbing sling

Pallet truck

PVC plate (to lay out for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Using pallet truck to shift out machine and equipment from factory premise

- Transport to loading area

- Using lorry crane to hoist up machine

- Load up machine onto lorry chassis

- Secured with strapping belt

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175