# METHOD OF STATEMENT

Date: 20th Aug 2020

Description of work

To shift out equipment from 5th floor to loading area via cargo lift, using forklift to fork up and load onto wooden base for packing, after packing load onto lorry chassis and deliver to another location, unload, shift and position into premise at ground floor

Spec

1 unit X-ray Inspection System GE Phoenix Vtomex S240

Dim: 210 x 160 x 210 cm

Wt: 2, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

25/08/2020 (Tuesday)

Time of Operation

09:45 am

Location of Operation

#### Collection: 50 Science Park Rd – Philip Morris International Research Laboratories Pte Ltd

Delivery: 5 Jalan Kilang Barat #01-02 – Petro Centre

Equipment Use

2 x 633 lorry loader

1 x 4.5ton forklift

Pallet truck/rollers

Plywood (for epoxy floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Access to be restricted to the site area, be cordoned off with safety cones, red/white tape

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman and forklift operator will be involved in this operation

# JOB SCOPE

- To lay out plywood for epoxy floor protection

- Shift out machine from 5th floor to loading area via cargo lift

- Using forklift to fork up machine and onto wooden base

- Pack to form a case

- After packing, load case onto lorry chassis

- Secured case with strapping belt

- Deliver to another location

- Unload case to the ground

- Uncrate case

- Fork up and detach machine from base

- Using rollers/pallet truck to transport machine into premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175