# METHOD OF STATEMENT

Date: 06th Apr 2022

Description of work

To shift out 1 unit equipment to loading, load onto wooden for packing, after packing, load up onto lorry chassis

Equipment spec

1 unit Nikon V12 Projector

Est wt: 600 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/04/22

Time of Operation

09:45 am

Location of Operation

#### 3 Toh Guan Rd Esat

#### Knight Auto Precision Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

Customer reach truck

Pallet truck

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

# JOB SCOPE

- Deploy lorry loader at designated loading area

- To shift out equipment to loading bay

- Lay out wooden base for packing

- Using customer reach truck to load equipment onto wooden base

- Pack to form a case

- After packing, load onto lorry chassis

- Secured case with strapping belt

- Deliver case to Eric Wong warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175