# METHOD OF STATEMENT

Date: 01st Apr 2022

Description of work

To shift out 1 unit equipment from 2nd floor via cargo lift to loading, load onto wooden for packing, after packing, load up onto lorry chassis and deliver to another location

Equipment spec

Epson IC Test Handler

Wt: 1, 300 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

04/04/22

Time of Operation

09:45 am

Location of Operation

#### 3 Toh Guan Rd Esat

#### Knight Auto

Equipment Use

1 x 633 lorry loader

1 x forklift

1 set webbing sling

Pallet truck

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

# JOB SCOPE

- Deploy lorry loader at designated loading area

- To shift out machine from 2nd floor to loading area via cargo lift

- Lay out wooden base for packing

- Using forklift to fork up and load machine onto wooden base

- Pack to form a case

- After packing, load case onto lorry chassis

- Secured case with strapping belt

- Deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175