# METHOD OF STATEMENT

Date: 09th June 2022

Description of work

To shift out 1 unit machine to loading area, load up onto lorry chassis using loading bay ramp and deliver to another location

Spec

1 unit X-Ray machine

Dim: 1.9 x 1.6 x 2 m

Wt: 2, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/06/22

Time of Operation

09:45 am

Location of Operation

#### 6 Fusionopolis Way, Synthesis #B1-26

#### National Metrology Centre

Equipment Use

1 x 633 lorry loader

1 x 3ton battery forklift

1 set wire/webbing sling

Pallet truck

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lorry loader and forklift operator will be involved in this operation

# JOB SCOPE

- Deploy lorry loader at designated loading area

- To lay out canvas/metal sheet along pathway for floor protection

- To shift out machine from production to loading area

- Load up machine onto lorry chassis using loading bay ramp up

- Secured machine with strapping belt

- Transport to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175