# METHOD OF STATEMENT

Date: 22nd Dec 2023

Description of work

To shift out equipment to loading area, using forklift to fork up onto lorry chassis and deliver to another location

Equipment Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

05/01/24

Time of Operation

09:30 am

Location of Operation

#### 3 Cleantech Loop, Cleantech II

#### Advanced Remanufacturing & Technology Centre

Equipment Use

1-2 x 633 lorry loader

1 x 2.5ton forklift

1 set wire/webbing sling

Pallet truck

Canvas, metal plate, PVC sheet (to lay out for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Lorry loader to hoist down forklift from lorry chassis

- To shift out equipment from 3rd & 5th floor to loading area via cargo lift

- Using forklift to fork up equipment

- Load up equipment onto lorry chassis

- Secured equipment with strapping belt prior to moving out of site premise

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175