# METHOD OF STATEMENT

Reference No: EW/Q/0863/23

Date: 15th Aug 2023

Description of work

To shift out machine to loading area, using forklift to fork up and load onto lorry chassis and deliver to another location

Spec

1 unit of 3 item Laser EOSM270 3D Printer

Item 1: 210 x 95 x 190 cm / Wt: 1, 100 kg

Item 2: 80 x 90 x 145 cm

Item 3: 55 x 55 x 70 cm

1 unit Nabertherm Furnace

Dim: 90 x 90 x 140 cm

Wt: 260 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

24/08/23

Time of Operation

09:40

Location of Operation

#### 180 A.M.K Ave 8

Nanyang Polytechnic, Blk S

Equipment Use

1 x 633 lorry loader

1 x 3ton forklift

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lifting supervisor, riggers, signalman, lorry loader & forklift operator will be involved in this operation

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Lorry loader to hoist down forklift from lorry chassis

- To shift out machine from workshop to loading area

- Using forklift to fork and load up machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175