# METHOD OF STATEMENT

Date: 15th Aug 2019

Description of work

To shift out 2 unit machine with accessories from production room to loading area, hoist and load onto wooden base for packing of total 8 cases, after packing stuff onto 1 x 40ft FR,

1 x 20ft FR and 1 x 40ft GP container

Machine Spec

Horizontal CNC Lathe/Integrex E500

Case 1: 905 x 344 x 366 cm / Wt: 30, 700 kgs

Case 2: 379 x 173 x 332 cm / Wt: 3, 350 kgs

Case 3: 290 x 129 x 129 cm / Wt: 450 kgs

Case 4: 130 x 130 x 259 cm / Wt: 600 kgs

Case 5: 744 x 195 x 197 cm / Wt: 3, 200 kgs

1 unit CTT4500

Case 1: 701 x 267 x 323 cm / Wt: 20, 800 kgs

Case 2: 130 x 130 x 259 cm / Wt: 650 kgs

Case 3: 599 x 204 x 202 cm / Wt: 2, 650 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBC

Time of Operation

TBC

Location of Operation

#### No 11 Tuas South Ave 12

#### Halliburton Completions MFG Pte Ltd

Equipment Use

1 x 100 ton mobile crane

1 x 633 lorry loader

1 x 3 ton battery forklift

Air rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert air rollers under machine bed

- Shift out machine from production area

- Transport machine to designated hoisting/packing area

- Lay out and stand by wooden base

- 100 ton mobile crane to hoist up machine and load onto wooden base for packing

- Pack to form a case

- After packing, hoist up case and load onto FR container

- Secured machine with choking wood and lashing wire

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175