# METHOD OF STATEMENT

Date: 22nd Mar 2023

Description of work

To shift out 2 unit machine with accessories from production room to loading area, fork up and load onto wooden base for packing, after packing stuff onto 2 x 40ft HQ container

Machine Spec

1 unit Mazak M5N CNC Lathe

Dim: 500 x 180 x 195 cm

Wt: 6, 600 kg

1 unit Mazak Power Master CNC Lathe

Dim: 580 x 220 x 215 cm

Wt: 9, 300 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

23/03/23

Time of Operation

10:00 am

Location of Operation

10 Gul Circle

#### OMS Oilfield Services Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 10ton forklift

Rollers/pallet truck

Wire sling

Hydraulic/hand jack

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine bed

- Shift out machine from production area

- Transport machine to designated packing area

- Lay out wooden base for packing

- Using 10ton forklift to fork up machine

- Load machine onto wooden base

- Secured and vacuum pack the machine

- After packing, load up onto container

- Secured machine with choking wood and lashing wire

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175