# METHOD OF STATEMENT

Date: 16th Apr 2024

Description of work

To shift out 1 unit machine with accessories from production room to loading area, fork up and load onto wooden base for packing, after packing stuff into 1 x 40ft HQ container

Machine Spec

6 cases of 1 unit Nanojet machine

Case 1: 4150 x 2240 x 2550 mm / Wt: 5, 800 kg

Case 2: 1000 x 1000 x 1360 mm / Wt: 320 kg

Case 3: 1500 x 1400 x 1670 mm / Wt: 350 kg

Case 4: 1000 x 1000 x 900 mm / Wt: 1, 100 kg

Case 5: 1770 x 1770 x 1640 mm / Wt: 1, 650 kg

Case 2: 1200 x 1200 x 1500 mm / Wt: 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

25/04/24

Time of Operation

10:00 am

Location of Operation

6 Seletar Aerospace Link

#### GE Aviation

Equipment Use

633 lorry loader

1 x 10ton forklift

Rollers/pallet truck

Webbing sling

Hydraulic/hand jack

Woodblock

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine bed

- Shift out machine from production area

- Transport machine to designated packing area

- Lay out wooden base for packing

- Using 10ton forklift to fork up machine

- Load up machine onto wooden base

- Pack to form a case

- After packing, load up cases into container

- Secured cases with choking wood and strapping belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175