# METHOD OF STATEMENT

Date: 05th Aug 2024

Description of work

To shift out 1 unit machine with accessories from production to loading bay, hoist up onto low loader, transfer to level 1 for packing, after packing load up onto estimate 1 x 20ft FR and 1 x 40ft HQ container

Machine Spec

1 unit DMU80 FD Duo Block

Main machine: 530 x 315 x 310 cm / Wt: 23, 500 kg

Chip conveyor: 320 x 120 x 155 cm

Filter: 195 x 185 x 170 cm

Chiller: 155 x 110 x 175 cm

Transformer: 85 x 75 x 100 cm

Mist collector: 150 x 100 x 80 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Time of Operation

09:45 am

Location of Operation

#### Blk 16B Tuas Ave 1 #02-40/42, JTC Space @ Tuas

Rohag

Equipment Use

110ton lorry loader

2 x 633 lorry loader

1 x low loader

Hydraulic jack/woodblock

Rollers/pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from production to loading bay

- Riggers to secured sling to crane hook

- Secured sling to rigging point of load to be lifted

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto low loader

- Release all sling from rigging point of machine

- Secured machine with strapping belt

- Transport machine to level 1

- Hoist down machine from lorry chassis

- Load machine onto wooden base

- Pack to form case

- After packing load up on container

- Secured case with chocking wood and lashing belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175